Analog IC Design Homework 3

學號: 102061112

姓名: 謝博楊

1.

Schematic:

(a) To find a proper Vb to let the drain current of M3=20uA, I connect M3 with a 20uA current source, and Vb=VGS.

from .lis file:

Vb=VGS is about 489.5mV, I choose 0.49

|Av| = gm*(ro//RD), so if we want to raise the gain, we expect (1) gm to be larger, which means larger W

(2) ro to be larger, which means larger L, but ro//RD, so the impact of L is narrowed. Thus, W is my trial priority.

(3) RD to be larger, which RD = 100k.

And my design is (W/L)M1 = (W/L)M2 = 84u /2.8u.

from .lis file:

The voltage gain= 25.3dB > 20dB:

(b) (W/L)M2 becomes 92.4u/2u.

(i) Without Cp:

|Av| = 25.3dB, |Acm| = 52.1dB

At low freq. CMRR = |Av / Acm| = 25.3dB - (-52.1dB) = 77.4dB

from the figure, -3dB bandwidth without Cp = 2.44M.

(ii) With Cp from P to ground:

|Av| = 25.3dB, |Acm| = 52.1dB

At low freq. CMRR = |Av / Acm| = 25.3dB - (-52.1dB) = 77.4dB is same as that without Cp.

from the figure, -3dB bandwidth with Cp = 1.73M.

Red curve: CMRR with Cp ; Blue curve: CMRR without Cp

(iii) Comment:

The common-mode to differential conversion becomes significant at high frequencies, since ro3 is shunted by Cp.

ACM-DM = -∆gmRD(gm1+gm2)(ro3//Cp)+1 and at high frequencies, Z(Cp) = 1/jwCp becomes smaller, so ro3//Cp becomes smaller, and ACM-DM becomes larger, resulting in smaller CMRR.

I compare the -3dB bandwidth , and the one without Cp is bigger than the one with Cp. It's reasonable since CMRR with Cp decrease faster, resulting in smaller bandwidth.

(c)

To calculate Vin,CM for all MOS saturation:

from .lis file

-> 0.43025+(0.49-0.44345)≦Vin,CM≦1.8-100k*20u2+0.42665

-> 0.4768≦Vin,CM≦1.22665

And from simulation, 0.36≦Vin,CM≦0.91

So combine above results, our desire IMCR is 0.4768≦Vin,CM≦0.91

(d)

The maximum differential input that the circuit can handle:

ΔVin = 2Id3μnCoxW/L = 2 (VGS1-VTH) = 2 (0.43-0.426) =0.00565V

Above figure is the simulation, and find the one with all MOS in saturation from .lis file(shown below),

So the input differential maximum is 0.9035-0.8965=0.007V

and the input differential range ΔVinis from 0 to 0.0035.

It's reasonable that it's smaller than calculation, since we want all MOS to be saturation.

(e)

Schematic:

The gain of using ideal voltage source to generate Vb:

Green curve: TT    Yellow curve: FF    Blue curve: SS

The gain of using current mirror to generate Vb:

Green curve: TT    Yellow curve: FF    Red curve: SS

Comment:

(i) For TT corner, both methods generating bias voltage will have all MOS in saturation, and reach a voltage gain of 25.3.

(ii) For FF corner, when using current mirror, we can still get a voltage gain of 25.2 and all MOS in saturation at the same time.

But when using ideal voltage source, the gain |Av| can reach 37.5, but not all MOS in saturation.

from .lis file

(iii) For SS corner, when using current mirror, we can still get a voltage gain of 25.2 and all MOS in saturation at the same time.

But when using ideal voltage source, the gain will drop to 20.5 and not all MOS in saturation.

from .lis file

(iv) We can observe that using current mirror to generate bias voltage is better in all three TT, FF, SS corners, since when operating in different corners, the parameters changed, so if we use the same ideal voltage source, it might not operate in saturation.

2.

Schematic:

(a)

We want Iout to be 16uA and Iref is 4uA, so we want

4(W/L)1,2 = (W/L)3,4.  I use the same length for all of the transistors so as to minimize errors due to the side diffusion of the source and drain areas, and make the multiple finger of 4.

For M1, M2 in saturation, VGS2+(VGS1-VTH1) ≦ Vb ≦VGS1+VTH2

(b)

M6 is guaranteed to be in saturation, since it's gate and drain are connected. But with all transistors in saturation region, M5 can only be in linear region, because Vin1 = VGS5 = VDS5 + VGS6, VGS6 > Vthand thus VGS5 - VDS5 > Vth.

So we can calculate the current:

For M6: ID = 12μnCox(W/L)6Vov2

For M5: ID = 12μnCox(W/L)5(2(Vgs-Vth) Vov-Vov2)

After calculation, we found that (W/L)6 is equal to (W/L)5 times 3, so I design (W/L)5 = 2u/4.5u and (W/L)6 = 2u/1.5u.

And ID4 can reach 16u, same as the result of giving bias voltage. And there is about 40mV difference from the original one, but I reckon that a roughly 5% difference is acceptable.

(c)

Figure of output current when using bias generation circuit:

Green curve: TT    Yellow curve: FF    Blue curve: SS

Figure of output current when using ideal voltage source:

Green curve: TT    Yellow curve: FF    Blue curve: SS

Comment:

(i) Using bias generation circuit, we have Iout close to 16uA, and from .lis file, we found only M5 in linear region as expected.

(ii) Using ideal voltage source, we have Iout close to 16uA as well, we concern that there might be transistor other than M5 not being in saturation. But in this case, from .lis file(shown below), we found it alright.

TT:

FF:

SS:

(d) M6 is guaranteed to be in saturation, since it's gate and drain are connected. But with all transistors in saturation region, M5 can only be in linear region, because Vin1 = VGS5 = VDS5 + VGS6, VGS6 > Vthand thus VGS5 - VDS5 > Vth.

(e)

Vin1 = the voltage with red circle in the figure above = Vov(M3)+(Vov+Vth)(M4) = 2Vov+Vth